Page 1 - What You Built

The problem is autonomous rendezvous, proximity operations, and docking with a non-cooperative GEO-class target in the last 15 meters, where the target can tumble, the dock interface is only intermittently observable, and millimeter-level port closure is not enough unless cone, clearance, lateral, velocity, alignment, and hard-capture gates are all satisfied simultaneously.

The stack is a Python reference simulation plus a C flight-software implementation for terminal RPOD: relative navigation, attitude estimation/control, mode management, Lambert/prox-ops sequencing, terminal port tracking, flash-lidar/dock-port handoff, spin synchronization, capture gating, watchdogs, packet validation, actuator command limiting, and desktop SIL/C-in-loop validation.

| Key Numbers |  |
| --- | --- |
| Docking baseline | 20 / 20 successful dockings |
| Mean total DV | 1.49 m/s |
| P95 total DV | 1.93 m/s |
| Mean time to dock | 1.78 hr |
| Final alignment | 7.33 deg mean; 9.21 deg max in the 20-run baseline |
| Current maturity | TRL 3-4: validated in simulation/SIL; HIL and flight hardware still pending |

# Page 2 - Architecture Diagram + Stack Overview

| Inputs | Flight Software Core | Outputs |
| --- | --- | --- |
| Orbital state IMU / gyro Mag, sun, star Range + camera Dock-port flash lidar Vehicle config | Packet validation + freshness TH-EKF relative navigation MEKF attitude estimation Chief pose + port tracker Mode manager + RPOD guidance ADCS + spin sync Capture gate + watchdogs | LVLH accel command RW torque command MTQ dipole command Mode/capture telemetry Watchdog flags CommandFrame ABI |
| Sensor truth and packetized measurements enter from the left. | The C flight loop estimates state, chooses mode, computes guidance/control, and gates capture. | Bounded actuator commands and health telemetry leave on the right. |

Sensor/packet layer: Validates ABI version, checksum, timestamps, and freshness before data can affect guidance.

Estimation: Runs relative translational EKF, attitude MEKF/QUEST, full-body pose estimation, and dock-port tracking.

Mode manager: Sequences DETUMBLE, SUN_ACQUISITION, FINE_POINTING, RPOD, terminal, soft capture, and docked states.

RPOD guidance: Executes prox-ops, terminal closing, port-targeting, lost-target handling, and soft-capture hold logic.

ADCS / spin sync: Commands attitude and rate matching while respecting pose freshness and actuator limits.

Capture gate: Certifies soft and hard capture using range, relative velocity, alignment, cone, lateral, and body-clearance gates.

Command ABI: Emits actuator commands and telemetry through a target-safe CommandFrame interface.

# Page 3 - Validation Evidence

## Monte Carlo Baseline

| Run | Trials | Success | Mean DV | P95 DV | Dock Time | Final Align |
| --- | --- | --- | --- | --- | --- | --- |
| Python clean baseline | 20 | 20/20 | 1.49 m/s | 1.93 m/s | 1.78 hr mean | 7.33 deg mean |
| Python mixed stress | 300 | 296/300 | 1.464 m/s | 1.967 m/s | 1.844 hr mean / 2.620 hr p95 | 7.30 deg mean |
| C-in-loop clean | 300 | 300/300 | 1.066 m/s | 1.108 m/s | 1.323 hr mean | 2.13 deg mean |

## Failure Chain Analysis

Finite-body geometry and coupled contact dynamics removed fake latch wins: a zero port range is not enough unless alignment, cone, clearance, lateral, and velocity gates pass.

Camera FOV limits caused estimator starvation near capture; full-body PnP could coast while the port was physically well tracked.

Terminal sensor handoff fixed the dominant issue: direct dock-port / flash-lidar axis is prioritized near capture when it is valid.

Replay evidence: trials 251 and 275 recovered after port-axis priority; trial 261 remains a spin-sync / pose-flip edge case; trial 144 is treated as an ADCS-gate case.

## SIL Test Results

C unit tests and desktop build complete with all major FSW modules covered: estimators, guidance, capture gate, ADCS, pose, port tracker, packet ABI, and flight loop.

Closed-loop C-in-loop runs validate the C flight loop against a Python plant; nominal 300-run CIL baseline reached 300/300 docked.

Fault-injection coverage includes range dropout, camera spike/dropout, gyro bias/dropout, lidar dropout, stale/frozen/delayed packets, bad timestamps, and actuator saturation.

Long-soak and real-time SIL checks exist for timing, finite telemetry, covariance sanity, watchdog behavior, and command clamps; target HIL is not yet completed.

# Page 4 - Interface / Integration

## SensorFrame ABI

| What goes in | Time tick, relative state/range, gyro, magnetometer, sun vector, star tracker attitude/vector data, camera/pose observations, dock-port flash-lidar packet, sensor-valid flags, timestamps, ABI version, and checksum. |
| --- | --- |
| Validation | Each packet is checked for version, checksum, timestamp freshness, finite values, and per-sensor validity before use. |
| Purpose | Provides the C flight loop with a deterministic, target-safe measurement boundary for Python plant, SIL, and future HIL. |

## CommandFrame ABI

| What comes out | Bounded LVLH acceleration command, reaction-wheel torque command, magnetorquer dipole command, active FSW/RPOD modes, nav/attitude estimates, capture-gate state, watchdog flags, timing counters, and output-inhibit state. |
| --- | --- |
| Boundary behavior | Final actuator clamps/inhibits are applied at the output boundary so invalid or stale inputs fail into bounded commands rather than unbounded control. |

## Vehicle Configuration Parameters

| Chief target | GEO-class IS-1002-like target; a=42164 km, e=0.0003; mass about 3000 kg; body half-extents [0.8, 0.8, 0.5] m; solar half-span about 6 m; dock port on +Z face. |
| --- | --- |
| Deputy vehicle | 50 kg servicer; 1 N-class thrusters; inertia diag approx [4.167, 4.167, 3.000] kg m^2; body half-extents [0.30, 0.30, 0.40] m. |
| Timing | Inner ADCS loop 100 Hz, RPOD/outer guidance 10 Hz, terminal handoff near 10 m, port-sensor authority inside terminal capture range. |
| Capture gates | Soft entry around 30 deg alignment with port/range/vrel gates; strict hard capture below 10 deg with cone, lateral, body-clearance, and hold-time checks. |
| Not yet done | Flight hardware procurement, board-specific target build validation, measured MCU timing jitter/WCET, and Python plant to target hardware closed-loop over the real comms link. |